CERTIFICATE OF ANALYSIS

PRODUCT NAME:   FORMIC ACID – Ever Bright

DATE:      JUL.24, 2013

Batch No.：  MS2013223

| ITEM | SPECIFICATION | ACTUAL RESULT |
| --- | --- | --- |
| APPEARANCE: | COLORLESS CLEAR LIQUID WITHOUT THE SUSPENDED | COLORLESS CLEAR LIQUID WITHOUT THE SUSPENDED |
| PURITY: | 85.00%MIN | 85.6% |
| CHROMA(PT-CO): | 30 MAX | 10 |
| CHLORIDE(CL): | 0.006%MAX | 0.002% |
| SULPHATE(SO4): | 0.02%MAX | 0.001% |
| IRON: ( Fe ) | 0.0006%MAX | 0.0001% |
| EVAPORATION RESIDUE | 0.06 %MAX | 0.005% |
| CONCLUSION: | QUALIFIED |  |
| REMARKS: |  |  |

FORMIC ACID 85%

Structural formula ：HCOOH

Molecular formula：CH2O2

Methods of manufacture：

Formic acid is obtained by the hydrolysis of methyl formate. The carbon monoxide and methanol generated methyl formate under the effect of catalyst. Methyl formate mixed with deionized water hydrolyzes under certain temperature and pressure. Different concentrations of formic acid are generated through the separation of low concentration of the formic acid solution.

CH3OH+CO→HCOOCH3

HCOOCH3+H2O→HCOOH+CH3OH

Identified uses：

1. Leather, it is mainly used for tanning, leather   processing .                                                     2.Agriculture,silage fodder                                                                                  3. Rubber,The flocculant of natural rubber